EE 616 Preliminary Evidence

# Initial Toy Simulation Results for Vision Only Leader Follower Coordination

Kinematic baseline for faculty review before ROS 2 and Gazebo implementation

| STATUS Toy baseline completed | OWNER Neeraj Kumar Kanchani | LAST UPDATED September 16, 2026 |
| --- | --- | --- |

| Field | Details |
| --- | --- |
| Faculty reviewer | Professor Masudul Imtiaz |
| Course | EE 616-57, section 9189, four credits |
| Experiment type | Deterministic two-dimensional kinematic simulation |
| Trial set | 32 trials; four scenarios; eight recorded random seeds |
| Decision requested | Approve the measurement-only Gazebo baseline as the next implementation gate |

# 1 Result and Interpretation

The toy baseline completed all 32 deterministic trials without a collision sample. Median camera-geometry error was 0.011 m for range and 0.104 degrees for bearing. Median formation error was 0.009 m, and the forced occlusion case reacquired a measurement after 0.030 s. These values show that the experiment harness, estimator, controller, and logging contract operate together under simplified assumptions. They are not evidence of ROS 2, Gazebo, learned detection, or physical robot performance.

Figure 1  Aggregate toy baseline results with proposed project thresholds shown only as references

# 2 Experiment Design

| Element | Toy baseline implementation |
| --- | --- |
| Scenarios | Straight motion, S-curve motion, constant-radius turn, and a forced visual occlusion |
| Perception | Synthetic bounding-box height and center with pixel noise at a nominal 15 Hz camera rate |
| Estimation | World-frame constant-velocity Kalman filter using follower pose and camera-derived range and bearing |
| Control | Bounded unicycle range-and-bearing controller at 50 Hz with TRACK, PREDICT, and SAFE_STOP states |
| Evidence | Per-trial metrics, representative time series, fixed configuration, recorded seeds, and four figures |

# 3 Representative Behavior

Figure 2  True measured and filtered leader range during the representative forced-occlusion trial

Figure 3  Leader and follower world paths during the representative S-curve trial

# 4 Occlusion Handling and Limitations

Figure 4  Controller state through the forced visual occlusion

# Limits of This Evidence

The follower pose is available without an odometry error model, the leader has a known marker height, and the camera model generates bounding-box geometry directly. The simulation does not include detector misses outside the declared occlusion, image artifacts, variable inference latency, wheel slip, actuator dynamics, message delay, or GPU contention. The low errors are therefore expected and should not be compared with real deployment performance.

# Recommended Next Gate

I recommend reproducing only the calibrated range-and-bearing experiment in Gazebo next. That gate should compare camera-derived measurements against Gazebo ground truth across range, heading, lighting, and off-axis placement. I will not treat the detector, filter, or controller as validated until that evidence is reviewed.

# Approval Requested

Please confirm whether this toy baseline and experiment structure are sufficient to begin the measurement-only Gazebo implementation. I will preserve the same seeds, configuration manifest, CSV schema, figures, and acceptance criteria so the next result can be compared directly with this baseline.

Use footnotes for definitions, assumptions, or terms that need precision without interrupting the main narrative.